# NUTANIX MIB

The Simple Network Management Protocol (SNMP) enables administrators to monitor network-attached devices for conditions that warrant administrative attention. In the Nutanix SNMP implementation, information about entities in the cluster is collected and made available through the Nutanix MIB (NUTANIX-MIB.txt). The Nutanix MIB is divided into the following sections:

- Cluster information. These fields provide status information about the cluster as a whole.

- Software version information. These fields provide version information about the software packages that comprise the Controller VM.

- Service information. These fields provide status information about the services of the Controller VM.

- Disk information. These fields provide status information about the disks in the cluster.

- Controller VM resource information. These fields indicate how much CPU and memory capacity is available to a Controller VM.

- Storage pool information. These fields provide status information about the storage pools in the cluster.

- Container information. These fields provide status information about the disks in the cluster.

- Alerts information. These fields provide information about generated alerts that can be captured through the SNMP trap (or inform) mechanism.

The following table describes the Nutanix MIB fields. The table is subdivided by the MIB sections.

| Name | Description | Values |
| --- | --- | --- |
| Cluster Information Fields |  |  |
| clusterName | Displays the cluster name. | (name) |
| clusterVersion | Displays the cluster version number. This is the Nutanix core package version expected on all the Controller VMs. | (version number) |
| clusterStatus | Displays the current status of the cluster. | [started ¦ stopped] |
| clusterTotalStorageCapacity | Displays the total storage capacity of the cluster in bytes. | xxxxx (bytes) |
| clusterUsedStorageCapacity | Displays the number of bytes of storage used on the cluster. | xxxxx (bytes) |
| clusterIops | Displays the average I/O operations per second (IOPS) in the cluster. | xxxx (IOPS) |
| clusterLatency | Displays the average I/O latency (in milliseconds) in the cluster. | xxxx [ms] |
| Software Version Information Fields |  |  |
| svtControllerVMId | Displays the Nutanix Controller VM identification number. | (ID number) |
| svtNutanixBootstrap | Displays the Nutanix bootstrap software package version. | (version number) |
| svtNutanixInfrastructure | Displays the Nutanix infrastructure software package version. | (version number) |
| svtNutanixCore | Displays the Nutanix core software package version. | (version number) |
| svtNutanixToolchain | Displays the Nutanix toolchain software package version. | (version number) |
| svtNutanixServiceability | Displays the Nutanix serviceability software package version. | (version number) |
| svtLinuxKernel | Displays the Linux kernel version currently installed. | (version number) |
| Service Information Fields |  |  |
| sstControllerVMId | Displays the Nutanix Controller VM identification number. | (ID number) |
| sstControllerVMStatus | Displays the current status of the node. The values areUporDown, and if the node is up and this is the lead node for the Zeus service (see following field), it will have a second value ofZeusLeader. | [Up¦Down] |
| sstZeusStatus | Displays the current status of the cluster configuration library manager (called "Zeus") on the node. Status is displayed as a comma separated list of process IDs for the Zeus service. If no PIDs appear, Zeus is not running. | (PID list) |
| sstScavengerStatus | Displays the current status of the file cleanup manager (called "Scavenger") on the node. Status is displayed as a comma separated list of process IDs for the Scavenger service. If no PIDs appear, Scavenger is not running. | (PID list) |
| sstMedusaStatus | Displays the current status of the metadata database manager (called "Medusa") on the node. Status is displayed as a comma separated list of process IDs for the Medusa service. If no PIDs appear, Medusa is not running. | (PID list) |
| sstPithosStatus | Displays the current status of a transactional key-value database manager (called "Pithos") on the node. Status is displayed as a comma separated list of process IDs for the Pithos service. (If no PIDs appear, Pithos is not running.) | (PID list) |
| sstStargateStatus | Displays the current status of the data receiving and processing manager (called "Stargate") on the node. Status is displayed as a comma separated list of process IDs for the Stargate service. (If no PIDs appear, Stargate is not running.) | (PID list) |
| sstChronosStatus | Displays the current status of the task scheduling service manager (called "Chronos") on the node. Status is displayed as a comma separated list of process IDs for the Chronos service. (If no PIDs appear, Chronos is not running.) | (PID list) |
| sstCuratorStatus | Displays the current status of the system overseer manager (called "Curator") on the node. Status is displayed as a comma separated list of process IDs for the Curator service. (If no PIDs appear, Curator is not running.) | (PID list) |
| sstPrismStatus | Displays the current status of the user interface manager (called "Prism") on the node. Status is displayed as a comma separated list of process IDs for the Prism service. If no PIDs appear, Prism is not running. | (PID list) |
| sstAlertManagerStatus | Displays the current status of the alert manager on the node. Status is displayed as a comma separated list of process IDs for the alert manager service. If no PIDs appear, the alert manager is not running. | (PID list) |
| sstStatsAggregatorStatus | Displays the current status of the statistics aggregator on the node. Status is displayed as a comma separated list of process IDs for the StatsAggregator service. If no PIDs appear, the StatsAggregator is not running. | (PID list) |
| sstSysStatCollectorStatus | Displays the current status of the system statistics collector on the node. Status is displayed as a comma separated list of process IDs for the SysStatCollector service. If no PIDs appear, the SysStatCollector is not running. | (PID list) |
| Disk Information Fields |  |  |
| dstDiskId | Displays the disk identification number. The number is unique for each disk. | (ID number) |
| dstControllerVMId | Displays the Nutanix Controller VM identification number. | (ID number) |
| dstSerial | Displays the disk serial number. | (serial number) |
| dstNumRawBytes | Displays the physical storage capacity (number of raw bytes) on the device. | xxxxx [bytes] |
| dstNumTotalBytes | Displays the usable storage (number of usable bytes) on the device through its file system. | xxxxx [bytes] |
| dstNumFreeBytes | Displays the available storage (number of free bytes) on the device through its file system for non-root users. | xxxxx [bytes] |
| dstNumTotalInodes | Displays the total number of usable inodes on the device through its file system. | xxxxx [inodes] |
| dstNumFreeInodes | Displays the total number of available (free) inodes on the device through its file system for non-root users. | xxxxx [inodes] |
| dstTemperature | Displays the Fusion IO disk temperature in degrees centigrade. | xx [degrees] |
| Controller VM Resource Information Fields |  |  |
| crtControllerVMId | Displays the Nutanix Controller VM identification number. | (ID number) |
| crtMemory | Displays the total memory available to the Controller VM in bytes. | xxxxx [bytes] |
| crtNumCpus | Displays the total number of CPUs allocated to the Controller VM. | (number) |
| Storage Pool Information Fields |  |  |
| spitStoragePoolId | Displays the storage pool identification number. | (ID number) |
| spitStoragePoolName | Displays the storage pool name. | (name) |
| spitTotalCapacity | Displays the total storage pool capacity in bytes. | xxxxx [bytes] |
| spitUsedCapacity | Displays the used storage pool capacity in bytes. | xxxxx [bytes] |
| spitIOPerSecond | Displays the current number of I/O operations per second (IOPS) for this storage pool. | (number) |
| spitAvgLatencyUsecs | Displays the average I/O latency for this storage pool in microseconds. | xxx [ms] |
| Container Information Fields |  |  |
| citContainerId | Displays the container identification number. | (ID number) |
| citContainerName | Displays the container name. | (name) |
| citTotalCapacity | Displays the total container capacity in bytes. | xxxxx [bytes] |
| citUsedCapacity | Displays the used container storage in bytes. | xxxxx [bytes] |
| citIOPerSecond | Displays the current number of I/O operations per second (IOPS) for this container. | (number) |
| citAvgLatencyUsecs | Displays the average I/O latency for this container in microseconds. | xxx [ms] |
| Alerts Information Fields |  |  |
| ntxAlertCreationTime | Displays when the alert was created. The value is the number of seconds since the UNIX epoch (01/01/1970). | xxxxx [seconds] |
| ntxAlertDisplayMsg | Displays the alert message text. For a list of alert messages and corrective actions, see Alerts. | (text string) |
| Table: Nutanix MIB Fields |  |  |